### Vehicle Inspection Process

All drivers are required to follow the Vehicle Inspection Process before starting their driving time each day. By adhering to these steps, drivers can establish a consistent routine and ensure a thorough pre-trip inspection, minimizing the risk of missed checks.

### Step 1: General Vehicle Condition

- Conduct an overall assessment of the vehicle.

- Look for obvious damage or fluid leaks under and around the vehicle.

### Step 2: Engine Compartment Inspection

Before lifting the hood, ensure your parking brake is set or the wheels are chocked. Inspect the following:

- Fluid Levels: Battery, automatic transmission, windshield washer fluid, power steering fluid, coolant, and engine oil.

- Belts and Hoses: Check for wear, cracks, or loose connections.

- Leaks: Look for fluid leaks around the engine.

- Electrical Wiring: Verify proper insulation and secure connections.

### Step 3: Cab Inspection

Enter the cab using three points of contact and ensure the vehicle is secure:

- ABS Light: Verify it turns on and off as expected.

- Safety Belt: Test for proper functionality and wear.

- Gauges and Warning Lights: Check for voltmeter, oil/air pressure, and coolant/oil temperature warnings. These lights should turn off quickly.

- Controls: Ensure the brake controls, lights, and horns operate correctly.

- Emergency Equipment: Confirm the presence of:

  - Three reflective triangles

  - Spare fuses

  - Emergency and tool kits

  - Tire changing equipment or chains

  - Fire extinguisher (charged and accessible)

  - Vehicle documents (current and in the cab)

### Step 4: Engine and Light Functions

- Turn off the engine.

- Test low-beam headlights and four-way emergency flashers.

- Exit the vehicle using three points of contact and keep the key in your pocket for safety.

### Step 5: Walkaround Inspection

Perform a 360-degree inspection of the vehicle:

- Lights and Reflectors: Verify all lights and reflectors are functional, clean, and correctly colored.

- Tires and Wheels:

  - Check tire inflation and condition.

  - Inspect lug nuts for rust or looseness.

  - Confirm no leaks around wheels and hubs.

- Vehicle Body: Look for any visible damage to frames, cross members, and splash guards.

- Axles and Suspension: Check axles, springs, U-bolts, shackles, and shock absorbers for wear or damage.

- Fluids and Fuel:

  - Ensure oil and transmission fluid levels are adequate.

  - Verify the fuel tank is secure and leak-free.

- Exhaust System: Confirm no leaks or interference with other parts.

- Batteries and Wiring: Check batteries for charge and inspect wiring for damage or loose connections.

### Step 6: Signal Lights Check

- Test signal lights by cycling through low and high beams, turn signals, and brake lights.

- Press the brake pedal to ensure the stop light works.

- Test the reverse lights as the final step.

### Step 7: Brake System Test

Perform a thorough check of the parking, service, and hydraulic brakes:

- Parking Brakes:

  - Set the parking brake and release the trailer brakes.

  - Attempt to move forward in a low gear. The vehicle should remain stationary.

  - Repeat for the trailer parking brakes while releasing the power unit brakes.

- Service Brakes:

  - Drive slowly (under 5 mph) and apply the brakes firmly.

  - If the brakes delay or the vehicle pulls to one side, seek professional inspection.

- Hydraulic Brakes:

  - Pump the brake pedal three times, then press down firmly.

  - If the pedal continues to move, it could indicate a fluid leak. Have it inspected immediately.